## Erosion and Sediment Control Model Code of Practice (Instream Works)

Compliance with a given Performance Criterion can only be achieved by:

- complying with the Acceptable Solution; or

- formulating an alternative solution which complies with the Performance Criterion, or is shown to be at least equivalent to the acceptable solutions; or

- a combination of (i) and (ii).

Unless otherwise indicated, all outcomes listed within the Acceptable Solution must be satisfied in order to comply with the Acceptable Solution.

Attachment A forms part of this Code.  The Attachment provides essential information and requirements not otherwise provided within the Code.

In the event of a conflict over the desired outcome of a Performance Criterion or an Acceptable Solution, then the outcome shall be that which best achieves the objective of the Code, that being:

To protect the environment while allowing for development that improves the total quality of life, both now and in the future, in a way that maintains the ecological processes on which life depends.

To achieve this objective a person must not carry out any activity that causes, or is likely to cause, environmental harm unless the person takes all reasonable and practicable measures to prevent or minimise the harm.

In assessing all reasonable and practicable measures, appropriate consideration must be given to:

- the nature of the potential harm; and

- the sensitivity of the receiving environment; and

- the current state of technical knowledge for the activity; and

- the likelihood of successful application of the various measures that might be undertaken; and

- the financial implications of the various measures relative to the type of activity.

The various recommendations presented in this guideline are an indication of what may be considered reasonable and practicable for the construction industry.

This model code of practice does not provide all the information necessary to adequately control soil erosion and sediment runoff in all situations.  Users of the Code should always make their own site-specific evaluation, testing and design, and refer to their own advisers and consultants as appropriate.

Specifically, the adoption of this model code of practice will not necessarily guarantee:

- compliance with any statutory obligations or licence conditions;

- avoidance of all environmental harm or nuisance.

| SITE PLANNING AND DESIGN |  |  |  |
| --- | --- | --- | --- |
| Performance Criteria |  | Acceptable Solution |  |
| P1 | Adequate data is obtained to allow appropriate site planning and design. | A1 | - The extent and complexity of data collection is commensurate with the potential environmental risk, and the extent and complexity of the instream disturbance. - Adequate soil data is obtained for the site to: - identify dispersive soils; - identify potential acid sulfate soils; - assess site revegetation/stabilisation works; - select and design ESC measures. |
| P2 | The design and layout of instream works minimise the risk of environmental harm occurring during the construction phase. | A2 | - Potential high-risk instream activities are identified during site planning. - Environmental risk, cost and safety are appropriately considered when determining the construction/maintenance process. - The design and layout of the instream works do not cause unnecessary soil disturbance if an alternative design or layout (which reduces the potential environmental harm) is available that achieves the same or equivalent project outcomes at a reasonable cost. - Site planning minimises the duration that any and all areas of soil will be exposed to the erosive effects of wind, rain and flowing water, in part through the progressive and prompt stabilisation of disturbed areas. - Instream sediment control measures are not employed if there is an appropriate off-stream sediment control process. - Development of the Erosion and Sediment Control Plan is an integral part of site planning. - Essential ESC control measures are appropriately integrated into the project’s design and costing. - Adequate space is provided for the installation and maintenance of essential ESC measures. - The number of temporary watercourse crossings is minimised. |
| P3 | The programming of instream works minimises the risk of environmental harm occurring during the construction phase. | A3 | - Instream disturbances are programmed to occur during the least erosive and environmentally damaging period of the year. - Instream works that require the construction of a weir or cofferdam, or an alteration in stream flow conditions, including flow velocity, bed roughness or flow rate, are not programmed for those periods when essential fish migration is expected to occur. |
| P4 | The design and layout of instream works minimise the risk of post-construction environmental harm. | A4 | - Flow velocities at the inlet and outlet of permanent drainage systems (e.g. stormwater pipes) are controlled to minimise ongoing erosion. - To the maximum degree reasonable and practicable, instream works are designed to minimise potential environmental harm during operational works and ongoing maintenance. |

| EROSION AND SEDIMENT CONTROL PLAN (ESCP) |  |  |  |
| --- | --- | --- | --- |
| Performance Criteria |  | Acceptable Solution |  |
| P5 | An Erosion and Sediment Control Plan (ESCP) is prepared prior to site disturbance that provides sufficient information to achieve the required environmental protection. | A5 | - The design standard of drainage, erosion and sediment controls (whether instream or off-stream) comply with the requirements of the relevant regulatory authority, or where such a standard does not exist, are designed in accordance with current best practice. - As a minimum, the standard of drainage, erosion and sediment controls are commensurate with the site conditions, (e.g. soil type, flow rate and erosion hazard), type of watercourse, local environmental values, and the type, cost and scope of the works. - The level of information and detail supplied in the ESCP is commensurate with the potential environmental risk, and the complexity of the proposed works; and is of sufficient clarity to allow on-site personnel to appropriately implement the plan. |
| P6 | The ESCP is prepared by, or under the supervision of, suitably qualified and experienced personnel. | A6 | - The qualifications and experience of the personnel preparing and/or supervising the preparation of the ESCP is commensurate with the potential environmental risk, and the extent and complexity of the soil disturbance. - On sites with a soil disturbance greater than 50m2, the ESCP is signed-off by a suitably qualified and experienced professional. - On sites with a flow diversion barrier extending over one-third of the channel width, or a temporary structure extending over the full channel width (e.g. watercourse crossing or instream sediment trap) the ESCP is signed-off by an engineer experienced in waterway hydraulics. |
| P7 | The ESCP remains relevant, at all times, to the current site conditions. | A7 | - The ESCP remains both effective and flexible, and is based on anticipated soil, weather, stream flow, and construction conditions (as may vary from time to time). - The ESCP is appropriately amended if the implemented works fail to achieve the objective of the ESCP, the required performance standard, or the State’s environmental protection requirements, or otherwise if there is the risk of serious or material environmental harm. |

| SITE ESTABLISHMENT |  |  |  |
| --- | --- | --- | --- |
| Performance Criteria |  | Acceptable Solution |  |
| P8 | Site personnel are provided with all necessary information prior to site establishment. | A8 | The Development Approval Conditions, Waterways Permit/Licence, Erosion and Sediment Control Plan, Monitoring and Maintenance Program, Site Rehabilitation Plan, and any other document required for the management of soil erosion and sediment control, are provided to the principal contractor prior to the commencement of land disturbing activities. |
| P9 | Appropriate personnel are engaged to monitor the site prior to commencement of site disturbance. | A9 | - Prior to the commencement of any instream disturbance, appropriately trained and experienced personnel are engaged to undertake regular ESC audits of the site. - Prior to commencement of site works, a “chain of command” in relation to the implementation, modification, and maintenance of ESC measures is established. |
| P10 | Site establishment does not cause unnecessary soil disturbance or environmental harm. | A10 | - No land-disturbing activities occur on the site until all appropriate ESC measures have been constructed in accordance with the ESCP and best practice erosion and sediment control. - All site office facilities and operational activities are located such that all effluent, including wash-down water, can be totally contained and treated within the site. |
| P11 | Site access is appropriately managed to minimise the risk of environmental harm. | A11 | - All reasonable and practicable measures are taken to ensure stormwater runoff from site access tracks and stabilised entry/exit systems, drains to an appropriate sediment control device. - Wherever reasonable and practicable, access tracks, whether temporary or permanent, are located a distance from the top of bank of at least 30m, or the width of the stream (measured at the top of the bank), whichever is the lesser. |

| SITE MANAGEMENT |  |  |  |
| --- | --- | --- | --- |
| Performance Criteria |  | Acceptable Solution |  |
| P12 | The work site is managed such that environmental harm is minimised. | A12 | - No land-disturbing activities (instream or off-stream) are undertaken prior to appropriate consideration being given to erosion and sediment control issues. - All works subject to an Erosion and Sediment Control Plan (ESCP) are carried out in accordance with the ESCP (as amended from time to time) unless circumstances arise where compliance with the ESCP would increase the potential for environmental harm as assessed by a recognised authority. - All ESC measures are installed, operated and maintained in accordance with current best management practice. - Land-disturbing activities are undertaken in such a manner that allows all reasonable and practicable measures to be undertaken to: - allow stormwater and stream flow to pass through the site in a controlled manner and at non-erosive flow velocities; and - minimise soil erosion resulting from wind, rain and flowing water; and - minimise the duration that disturbed soils are exposed to the erosive forces of wind, rain and flowing water; and - prevent, or at least minimise, environmental harm (including public nuisance and safety issues) resulting from work-related soil erosion and sediment runoff. - Site spoil is lawfully disposed of in a manner that does not result in ongoing soil erosion or environmental harm. |
| P13 | Those responsible for erosion and sediment control are appropriately trained and equipped. | A13 | Site managers and/or the nominated responsible ESC personnel achieve and maintain a good working knowledge of the correct installation and operational procedures of all ESC measures used on the site. |
| P14 | Disturbance to ESC measures by on-site personnel is minimised. | A14 | - On-site personnel are appropriately instructed and educated as to the purpose and operation of adopted drainage, erosion and sediment control (ESC) measures, and the need to maintain such measures in proper working order at all times. - Unnecessary disturbance to ESC measures by on-site personnel, sub-contractors and construction traffic (including site management and material delivery vehicles) is minimised. |
| P15 | The adopted ESC measures remain relevant at all times to the current site conditions. | A15 | - Performance of the site’s ESC measures is monitored in accordance with the site’s Monitoring and Maintenance Program. - The adopted erosion and sediment control measures are appropriately amended if site conditions significantly change, or are expected to significantly change, from those conditions assumed during development of the ESCP. - The adopted erosion and sediment control measures are appropriately amended if the implemented works fail to achieve the “objective” of the ESCP, or the required performance standard, or the State’s environmental protection requirements, or unacceptable environmental harm is occurring or is likely to occur. |
| P16 | The work site is appropriately prepared for imminent construction activities and weather conditions. | A16 | - Adequate supplies of drainage, erosion and sediment control, and relevant pollution clean-up materials, are retained on-site during the construction period. - Appropriate short-term drainage control measures (e.g. flow diversion around soil disturbances and recently opened trenches) are installed and operational prior to impending storms or increased stream flows. |
| P17 | Land disturbing activities do not cause unnecessary soil disturbance. | A17 | - Land disturbing activities do not cause unnecessary soil disturbance if an alternative construction process (that reduces potential environmental harm) is available that achieves the same or equivalent project outcomes at a reasonable cost. - The extent of unnecessary soil disturbance, including disturbances outside the designated work area, is minimised. |
| P18 | Damage to retained or protected vegetation is minimised. | A18 | - Prior to the commencement of land disturbing activities within any given area, all protected vegetation and significant areas of retained vegetation within that area, are appropriately identified to minimise the risk of disturbance to such areas. - No damage is allowed to occur to roots, trunk or branches of retained vegetation, unless under the direction of an appropriate Vegetation Management Plan. |
| P19 | Adopted work practices minimise the release of pollutants into receiving waters. | A19 | - Emergency and pollution control procedures are commensurate with the site conditions, local environmental values, and the type, cost, scope and complexity of the works. - All liquid chemicals, including petroleum products, that could potentially be washed or discharged from the site in association with sediment, are stored and handled on-site in accordance with relevant standards such as AS1940. - Adequate supplies of erosion control, sediment control, and pollution clean-up materials are retained on-site during the construction period. - Cement-laden runoff, concrete waste, and chemical products (including petroleum and oil-based products), are managed on-site in accordance with current best management practice. - All equipment is washed down (cleaned) well away from the water’s edge, and in a manner that prevents sediment-laden water entering the waters. - All non water-soluble pollutants washed or blown onto waters are collected and secured as soon as practicable. - All waste receptors are sealed and/or covered outside working hours to prevent the entry of water and vermin, or wind disturbance of the contained material. |
| P20 | Adopted work practices minimise the release of pollutants into tidal waters. | A20 | - No erodible material is stockpiled within 40m from the high tide mark. - Sediment deposition within the voids between natural and introduced rock located within the tidal zone is minimised. - All materials being transported by boats or barges are adequately secured during transportation. - Drip pans are placed under all vehicles and motorised equipment placed on docks, barges, or other structures that extend over water bodies, if the vehicle or equipment is expected to be idle for more than 1 hour. - All barges are fitted with watertight curbs or toe boards to contain spills and prevent materials, tools, and debris from leaving the barge. - All appropriate measures are deployed to provide secondary containment for any spills while materials and/or equipment are being transferred on and off barges to (e.g. floating sediment curtains). |
| P21 | Environmental harm, safety issues, and nuisance or damage to public and private property resulting from off-site sediment deposits, material spills, and/or the adopted ESC measures is minimised. | A21 | - Sediment and other material originating from the work area, or as a result of the transportation of materials to or from the work area, that collect on sealed roads, or within gutters, drains or drainage channels outside the immediate work area, is removed: - immediately if rain is occurring or imminent; or - immediately if considered a safety hazard; or - if items (i) or (ii) do not apply, as soon as practicable, but before completion of the day’s work. - The adopted ESC measures do not adversely affect drainage or flooding conditions within neighbouring properties. |
| P22 | Potential safety risks to site workers and the public as a result of ESC measures are minimised. | A22 | All stream flow diversion and ESC measures are installed and operated in a manner that does not cause a safety risk to the public or site personnel. |
| P23 | Potential harm to wildlife as a result of ESC measures is minimised. | A23 | - Disturbance to wildlife habitats is limited to the minimum necessary to complete the approved works. - Synthetic (plastic) reinforced fabrics are not placed within, or adjacent to, bushland areas, riparian zones and watercourses if such materials are likely to cause harm to wildlife or wildlife habitats. - The design of temporary instream structures does not adversely impact on terrestrial and aquatic passage along the waterway. - To the maximum degree reasonable and practicable, instream disturbances are programmed to occur during periods of least impact to fish migration. - Sediment traps, flow diversion systems and isolation barriers allow appropriate egress of wildlife where such wildlife could enter such areas. - Site rehabilitation procedures and outcomes are compatible with site conditions and local environmental values (including local wildlife). |

| SITE DISTURBANCE |  |  |  |
| --- | --- | --- | --- |
| Performance Criteria |  | Acceptable Solution |  |
| P24 | Potential environmental harm resulting from land clearing is minimised. | A24 | - All land clearing is conducted in accordance with State and local government Vegetation Protection and/or Preservation requirements and/or policies. - No instream disturbances are undertaken prior to development of a Vegetation Management Plan. - No instream soil disturbance occurs until the principal instream works are ready to commence. - Controls placed on the extent and duration of soil disturbance are commensurate with the potential erosion risk and/or erosion hazard. - To the maximum degree reasonable and practicable, disturbance to deep-rooted vegetation on slopes susceptible to mass movement is minimised, if not totally avoided. - Compliance with Performance Criterion P18. |
| P25 | Disturbance to natural watercourses is minimised. | A25 | - Disturbance to natural watercourses (including bed and bank vegetation) and their associated riparian zones is limited to the minimum necessary to complete the approved works. - The number, location, type and size of temporary watercourse crossing are such that the overall adverse impact on the environment is minimised. - All temporary watercourse crossings, including their approach roads, employ appropriate drainage, erosion and sediment controls to minimise sediment inflow into the watercourse. |
| P26 | Disturbance to tidal and intertidal areas including any associated riparian zones is minimised. | A26 | - Disturbance to aquatic vegetation, particularly seagrasses and mangroves, is minimised. - Vehicle/boat damage to seawalls (e.g. due to wave and wash conditions) is minimised. |

| SOIL AND STOCKPILE MANAGEMENT |  |  |  |
| --- | --- | --- | --- |
| Performance Criteria |  | Acceptable Solution |  |
| P27 | Maximum benefit is obtained from existing topsoil. | A27 | - The topsoil is managed (i.e. stripped, treated, stockpiled and reused) in accordance with the recommendations of an approved Vegetation Management Plan or similar. OR - Topsoil is stripped, stockpiled, placed, and where necessary treated, in accordance with current best practice. |
| P28 | Environmental harm caused by the temporary stockpiling of erodible material is minimised. | A28 | Stockpiles of erodible material are: - appropriately protected from wind, rain and surface flows in accordance with current best practice; and - located at least 2m from hazardous areas, retained vegetation; and - located up-slope of an appropriate sediment control system. |
| P29 | Exposed dispersive soils are managed such that the risk of ongoing soil erosion is minimised. | A29 | Construction details for drainage systems and bank stabilisation works within dispersive soil areas clearly demonstrate how these soils will be managed to prevent future erosion problems. |
| P30 | Exposed potential acid sulfate soils are appropriately managed. | A30 | - If acid sulfate soils conditions exist on site, then appropriate warnings are placed on the ESCP. - All exposed actual or potential acid sulfate soils are managed in accordance with current best practice. - On-site personnel involved in the disturbance of actual or potential acid sulfate soils are appropriately trained and/or supervised. |

| MANAGEMENT OF STREAM FLOW |  |  |  |
| --- | --- | --- | --- |
| Performance Criteria |  | Acceptable Solution |  |
| P31 | Temporary drainage control measures are designed, constructed and maintained to an appropriate standard. | A31 | - The standard of stream flow control complies with the requirements of the relevant regulatory authority, or where such a standard does not exist, flow controls are designed in accordance with current best practice. - The adopted stream flow control measures remain relevant, at all times, to the current and imminent site conditions. - Instream flow diversion structures are structurally sound during a 1 in 2 year ARI channel flow. - Wherever reasonable and practicable, isolation barriers do not isolate more than 30% of the channel width at any given time, otherwise not more than 50%, while channel flows are occurring. |

| DRAINAGE CONTROL |  |  |  |
| --- | --- | --- | --- |
| Performance Criteria |  | Acceptable Solution |  |
| P32 | Temporary drainage control measures are designed, constructed and maintained to an appropriate standard. | A32 | - The standard of drainage control complies with the requirements of the relevant regulatory authority, or where such a standard does not exist, drainage controls are designed in accordance with current best practice. - The adopted drainage control measures remain relevant, at all times, to the current and imminent site conditions. |
| P33 | Stormwater movement through the site is appropriately managed to minimise soil erosion. | A33 | - If the overbank drainage area up-slope of a soil disturbance exceeds 1500m2, and the average monthly rainfall exceeds 45mm, all stormwater discharged from this area (up to the design storm) is diverted around or through the soil disturbance in a manner that minimises soil erosion. - Appropriate drainage controls are installed above an exposed stream bank to minimise soil erosion on the bank. - Flow velocities within flow diversion channels and at the entrance and exit of all drainage structures (including Chutes, and Slope Drains) are controlled in such a manner that prevents soil erosion during all discharges up to the relevant design discharge. |
| P34 | Stormwater movement through the site is appropriately managed to minimise environmental harm. | A34 | - Overbank stormwater runoff passing around or through the work site does not cause erosion to the banks of water bodies. - All reasonable and practicable measures are taken to ensure stormwater runoff entering an area of soil disturbance is diverted around or through that area in a manner that minimises soil erosion and contamination of that water for all discharges up to the specified design discharge. - Adequate drainage controls (e.g. cross drainage systems and/or longitudinal drainage) are applied to access tracks to minimise erosion on, and sediment runoff from, such areas. |

| EROSION CONTROL |  |  |  |
| --- | --- | --- | --- |
| Performance Criteria |  | Acceptable Solution |  |
| P35 | Erosion control measures are designed, installed and maintained to an appropriate standard. | A35 | - The standard of erosion control complies with the requirements of the relevant regulatory authority, or where such a standard does not exist, erosion controls are designed in accordance with current best practice. - As a minimum, the type and degree of erosion control are commensurate with the expected site conditions, soil type, stream flow, potential environmental risk, and the type, cost and scope of the works. - The adopted erosion control measures remain relevant, at all times, to the current and imminent site conditions. |
| P36 | The control of soil erosion is given appropriate priority. | A36 | - Wherever reasonable and practicable, priority is given to the prevention, or at least minimisation, of soil erosion, rather than allowing soil erosion to occur and trying to trap the resulting sediment. - The existence of best practice sediment control measures within a given sub-catchment does not diminish the need for the application of current best-practice erosion control measures. |
| P37 | Soil erosion is minimised. | A37 | - Existing ground covers are protected from damage and retained as long as practicable. - Site activities are carried out in a manner that minimises the duration that any and all disturbed soil surfaces are exposed to the erosive forces of wind, rain and flowing water. - All temporary erosion control measures are appropriately anchored to the soil as appropriate for the expected flow conditions. - Mechanical equipment does not enter the channel if alternative equipment or construction procedures are available that would allow the works to be conducted from an overbank location. |
| P38 | Soil erosion resulting from stream flow is minimised. | A38 | - All reasonable and practicable steps are taken to apply best practice erosion control measures to completed channel works, or otherwise stabilise such works, prior to an anticipated increase in stream flow. - Bed and bank stabilisation and revegetation methods are appropriate for the expected stream flow conditions such that ongoing soil erosion is minimised. - Dispersive soils are either treated, or covered with a layer of non-dispersible soil (200mm minimum) before being covered with vegetation, rock, mulch, or erosion control blankets. |

| SEDIMENT CONTROL |  |  |  |
| --- | --- | --- | --- |
| Performance Criteria |  | Acceptable Solution |  |
| P39 | Sediment control measures are designed, installed, operated and maintained to an appropriate standard. | A39 | - The standard of sediment control complies with the requirements of the relevant regulatory authority, or where such a standard does not exist, sediment controls are designed in accordance with current best practice. - As a minimum, the type and degree of sediment controls are commensurate with the expected site conditions, soil type, stream flow, potential environmental risk, and the type, cost and scope of the works. - Instream sediment control measures are designed for the expected base flow (i.e. stream flow not affected by flood flows or storm runoff). - The adopted sediment control measures remain relevant at all times to the current and imminent site conditions. |
| P40 | Sediment contamination of instream waters is minimised. | A40 | - All reasonable and practicable measures are taken to prevent, or at least minimise, the release of sediment from overbank areas into waters. - Wherever reasonable and practicable, instream disturbances are managed in accordance with the following hierarchy: - minimise, if not totally avoid, direct contamination of stream flows (e.g. through the use of flow diversion systems and the appropriate timing of instream works); - treatment of sediment-laden water within off-stream sediment traps; - treatment of sediment-laden water within instream sediment traps. - A suitable off-stream sediment trap is placed down-slope of any off-stream soil disturbance prior to the disturbance occurring. - Appropriate stream flow and/or sediment controls are installed and made operational before any instream soil disturbance occurs. |
| P41 | Sediment displaced off site by vehicular traffic is minimised. | A41 | - Number of site entry/exit points is limited to the minimum practical number. - Site entry/exit points are appropriately designed and stabilised to minimise sediment being washed off the site or into adjacent waters. - Sediment-laden stormwater runoff from access tracks and stabilised entry/exit systems drains to an appropriate sediment control device. |
| P42 | Sediment-related environmental harm resulting from de-watering activities is minimised. | A42 | - Flow diversion barriers, or other appropriate systems, are used to minimise the quantity of watering entering excavations and trenches. - As a minimum, sediment control measures implemented for the control of sediment-laden discharge from de-watering activities are designed to satisfy current best practice. |

| SITE STABILISATION AND REHABILITATION |  |  |  |
| --- | --- | --- | --- |
| Performance Criteria |  | Acceptable Solution |  |
| P43 | Site rehabilitation, including site revegetation, is designed, installed and maintained to an appropriate standard. | A43 | - A Site Stabilisation Plan or similar is prepared and approved by the relevant regulatory authority prior to site establishment. - The standard of site rehabilitation complies with the requirements of the relevant regulatory authority or, where such a standard does not exist, complies with current best practice. - As a minimum, the type and degree of site rehabilitation is commensurate with the expected site conditions, soil type, stream flow, potential environmental risk, and the type, cost and scope of the works. |
| P44 | Site rehabilitation methods and procedures minimise the risk of environmental harm. | A44 | - Site revegetation (excluding temporary revegetation conducted for purposes of erosion control) is conducted in accordance with a Site Stabilisation Plan or similar, where such a plan exists. - Disturbed soil surfaces are appropriately stabilised to minimise the risk of short-term soil erosion. - All temporary ESC measures are removed and the land rehabilitated as soon as practicable after their use is no longer needed. |
| P45 | Site rehabilitation methods, procedures and outcomes are compatible with site conditions and local environmental values. | A45 | - The qualifications and experience of the personnel preparing and/or supervising the preparation of any Site Stabilisation Plan, Vegetation Management Plan, or similar, is commensurate with the potential environmental risk, and the extent and complexity of the works. - Plant selection and landscape design are compatible with identified environmental values. |

| SITE INSPECTION AND MONITORING |  |  |  |
| --- | --- | --- | --- |
| Performance Criteria |  | Acceptable Solution |  |
| P46 | A Monitoring Program is prepared by, or under the supervision of, suitably qualified and experienced personnel. | A46 | - A Water Quality Monitoring Program is prepared and approved by the relevant regulatory authority prior to site establishment. - The qualifications and experience of the personnel preparing and/or supervising the preparation of the Monitoring and Maintenance Program is commensurate with the potential environmental risk, and the extent and complexity of the works. |
| P47 | The performance of the site’s drainage, erosion and sediment control measures is regularly monitored. | A47 | - The extent and complexity of site monitoring (including water quality monitoring) is commensurate with the potential environmental risk, and the extent and complexity of the works. - A record is maintained of the site’s compliance and non-compliance with erosion and sediment control approval requirements. - All site monitoring data including environmental incidents, rainfall records, dates of water quality testing, testing results, and records of controlled water releases for the site, are kept in a register. |
| P48 | The site’s stream flow, drainage, erosion and sediment control measures remain relevant at all times to the current site conditions. | A48 | All stream flow and ESC measures are inspected by site personnel: - at least daily (when work is occurring on-site); - at least weekly (when work is not occurring on-site); - within 24-hours of expected rainfall; and - within 18-hours of a rainfall event of sufficient intensity and duration to cause runoff on the site. |

| SITE MAINTENANCE |  |  |  |
| --- | --- | --- | --- |
| Performance Criteria |  | Acceptable Solution |  |
| P49 | All ESC measures are maintained in proper working order at all times during their required operational life. | A49 | - All ESC measures are maintained in proper working order for the duration of the period in which their operation is required in order to satisfy the required treatment standard, and/or the objective of the ESCP. - All sediment control measures are maintained in accordance with the requirements of the relevant regulatory authority, or where such a standard does not exist, in accordance with current best practice. - As a minimum, the maintenance of all ESC measures is commensurate with the expected site conditions, and potential environmental risk. |
| P50 | The maintenance of ESC measures does not cause environmental harm. | A50 | All materials removed from ESC devices during maintenance or decommissioning, whether solid or liquid, is lawfully disposed of in a manner that does not cause ongoing soil erosion or environmental harm. |

## Attachment A

SITE PLANNING AND DESIGN

The intent of the Site Planning and Design section is to:

- Enable erosion and sediment control issues to appropriately influence the planning and design of instream works for the purpose of minimising their overall adverse environmental impact.

- Enable planners and designers to recognise that along with consideration of the operational phase of a development, appropriate consideration must be given to how something is to be constructed and maintained, and the potential adverse impacts of the construction and maintenance phases.

- Take all reasonable and practicable measures to actively avoid foreseeable soil erosion problems and associated environmental hazards during the construction phase.

The term “maintenance phase” refers to such activities as the de-silting of instream structures such culverts, stormwater pipes, and permanent instream sediment traps.

Acceptable Solution A1(a)

Data collection may include: soil testing, identification of potential site constraints, and development of a Conceptual Erosion and Sediment Control Plan (where such data and/or plans are considered reasonably necessary to enable appropriate site planning and design). Appropriate site planning and design refers to the aim of minimising the potential environmental harm (both during the construction and operational phases) of the instream works.

The “potential environmental risk” relates to the potential of a land-disturbing activity to cause harm, whether material, serious, reversible or irreversible, to an environmental value, including nuisance to a neighbouring property or person. The potential environmental risk is related, in part, to the assessed Erosion Hazard.

Acceptable Solution A1(b)

Data collection necessary to assist the design of site revegetation is outlined in Sections C3 and C9 of IECA (2008).

Acceptable Solution A2(a)

Construction activities that are deemed to represent a high to extreme erosion hazard include:

- Any disturbance of high to extreme hazard areas, or a problematic soil that could result in unmanageable soil erosion and/or environmental harm.

- Any construction or building activity, or procedure, that could potentially cause “serious” environmental harm.

- Any soil disturbance that could cause the transformation of significant quantities of potential acid sulfate soils (PASS) into actual acid sulfate soils (AASS), such as to cause “material” or “serious” environmental harm.

Acceptable Solution A2(f)

Ideally, Erosion and Sediment Control Plans (ESCPs) should be developed in close association with construction planning because the needs and limitations of the construction process represent an important component of the ESCP.  In theory, a construction process cannot be finalised without reference to an ESCP, and an ESCP cannot be finalised without knowledge of the construction process.

Acceptable Solution A2(g)

Essential ESC control measures includes any instream sediment control and flow diversion systems, and bank and overbank drainage, erosion or sediment control measures.

Acceptable Solution A2(h)

The most critical issue is ensuring sufficient space is available to construct and maintain all Sediment Basins and flow diversion systems.

Acceptable Solution A2(i)

“Temporary” watercourse crossings refer to those crossings constructed for use only during the construction phase.

Acceptable Solution A3(a)

Minimising the potential environmental harm can be achieved, in part, by scheduling major land disturbances, and disturbances to high and extreme erosion risk areas, for the least erosive periods of the year.

The least erosive period of the year is usually the period of lowest stream flow.  The least environmentally damaging period of the year usually relates to periods of no, or minimum, fish migration.  Refer to State fisheries authorities for advice.

Acceptable Solution A4(a)

Ongoing erosion problems can result from any of the following:

- changes to the volume, duration, frequency or rate of stormwater runoff;

- excessive (i.e. erosive) flow velocities;

- inappropriate distribution of flow velocities throughout the depth and width of flow discharged from a stormwater drain into a receiving water;

- inappropriate direction of flow discharged from a stormwater drain into a receiving water.

Acceptable Solution A4(b)

“Ongoing maintenance” refers to such activities as the de-silting of instream structures such culverts, stormwater pipes, and permanent instream sediment traps.

EROSION AND SEDIMENT CONTROL PLAN (ESCP)

The intent of this section is to ensure Erosion and Sediment Control Plans (ESCPs):

- are appropriate for the site conditions, which may vary from time to time;

- are prepared by, or under the supervision of, suitable personnel;

- are able to achieve the required design standard and environmental protection.

Acceptable Solution A5(a)

Such a clause shall not reduce the responsibility of applying and maintaining, at all times, all necessary sediment control measures in accordance with the sediment control standard.

Acceptable Solution A5(b)

Refer to A1(a) for discussion on “environmental risk”.

It is recognised that the degree of erosion and sediment control is related to the type, cost and scope of works in addition to the environmental risk.  This association is acknowledged within the terms of current best practice erosion and sediment control as defined within this document (2008 conditions).

Acceptable Solution A5(c)

On very minor works, such as regular council maintenance activities, or the installation of minor services, the ESCP may be represented by standard drawings prepared by the principle company/organisation as part of an in-house Code of Practice.  The key intent is to ensure that appropriate consideration is given to erosion and sediment control requirements before works commence.

For instream works with a soil disturbance greater than 50m2, the Erosion and Sediment Control Plan (including supporting documentation and construction specifications) must include:

- North point and plan scale.

- Site and easement boundaries and adjoining roadways.

- Construction access points.

- Site office, car park and location of stockpiles.

- Proposed construction activities and limits of disturbance.

- Retained vegetation including protected trees.

- General soil information and location of problem soils.

- Location of critical environmental values (where appropriate).

- Existing site contours (unless the provision of these contours adversely impacts the clarity of the ESCP).

- Final site contours including locations of cut and fill.

- General layout and staging of proposed works.

- Location of all drainage, erosion and sediment control measures.

- Full design and construction details (e.g. cross-sections, minimum channel grades, channel linings,) for all drainage and sediment control devices, including Flow Diversion Barriers and instream sediment traps.

- Construction specifications for adopted ESC measures (as appropriate).

- Site revegetation requirements (if not contained within separate plans).

- Site Monitoring and Maintenance Program, including the location of proposed water quality monitoring stations.

- Technical notes relating to:

- site preparation and land clearing;

- extent, timing and application of erosion control measures;

- temporary ESC measures installed at end of working day;

- temporary ESC measure in case of impending storms or elevated stream flows, or emergency situations;

- installation sequence for ESC measures;

- application rates (or at least the minimum application rates) for mulching and revegetation measures;

- legend of standard symbols used within the plans.

- Calculation sheets for the sizing of ESC measures.

- A completed Erosion and Sediment Control Plan checklist such as presented in (insert publication).

- Any other relevant information the regulatory authority may require to properly assess the ESCP.

The ESCP must clearly state that no land-disturbing activities shall occur on the site until all associated perimeter ESC measures, including flow diversion barriers, sediment traps and temporary drainage controls, have been constructed in accordance with the ESCP and current best practice erosion and sediment control procedures.

Acceptable Solution A6(a) & (b)

A suitably qualified and experienced professional is defined as a person with:

- training and/or qualifications in erosion and sediment control that are recognised by the regulatory authority; and

- professional affiliations with an engineering, environmental engineering, soil science, and/or scientific organisation (e.g. the International Erosion Control Association; Engineers Australia; Environment Institute of Australia and New Zealand; or the Australian Society of Soil Science Inc.) and

- at least 2 years experience in the management of erosion and sediment control that can be verified by an independent third party.

ESCPs for high-risk sites should be reviewed by a suitably qualified and experienced third party reviewer prior to its implementation.

The assessment and categorisation of high-risk sites may be defined by the relevant regulatory authority; otherwise, refer to the discussion in Chapter 3 and Appendix F of IECA (2008).

Acceptable Solution A6(c)

The intent is to ensure the adoption of appropriate design procedures for temporary instream structures, and to minimise the risk of avoidable harm to the waterway.

Acceptable Solution A7(a)

The timing and degree of ESC specified in the Erosion and Sediment Control Plan(s) needs to be appropriate for the given soil properties, expected weather conditions, and susceptibility of the receiving waters to environmental harm resulting from sediment-laden runoff.  Current (2008) best practice design standard of the drainage, erosion and sediment control measures are outlined in Chapter 4 of IECA (2008).

Acceptable Solution A7(b)

Additional and/or alternative erosion and sediment control measures must be implemented, and a revised Erosion and Sediment Control Plan (ESCP) must be prepared and submitted to relevant regulatory authority for approval (where required) in the event that:

- site conditions significantly change from those previously anticipated; or

- there is a high probability that serious or material environmental harm might occur as a result of sediment leaving the site; or

- the implemented works fail to achieve the adopted ESC standard, or the State’s environmental protection requirements; or

- site inspections indicate that the implemented works are failing to achieve the “objective” of this ESCP.

SITE ESTABLISHMENT

The intent of this section is to ensure that during site establishment:

- on-site personnel are provided with all necessary information to fully comply with all legal requirements, minimise environmental harm, and achieve the objective of the ESCP; and

- land disturbing activities proceed in a manner consistent with the objective of the ESCP.

Acceptable Solution A8

Supply of such material is relevant only to that material that exists, or is required to exist.

Acceptable Solution A9(a)

On low-risk site, ESC audits (including site inspections and water quality monitoring) may be performed by site personnel; however, as the risk of environmental harm increases, the need for third-party site inspections and water quality monitoring increases.

In reference to instream works, “low-risk sites” would include works conducted within dry-bed channels during periods when stream flow is highly unlikely.

Personnel undertaking ESC audits of a site must, collectively, have the following capabilities:

- an understanding of the local environmental values that could potentially be affected by the proposed works; and

- a good working knowledge of the site’s Erosion and Sediment Control (ESC) issues, and potential environmental impacts, that is commensurate with the complexity of the site and the degree of environmental risk; and

- a good working knowledge of current best practice ESC measures for the given site conditions and type of works; and

- ability to appropriately monitor, interpret, and report on the site’s ESC performance, including the ability to recognise poor performance and potential ESC problems; and

- ability to provide advice and guidance on appropriate measures and procedures to maintain the site at all times in a condition representative of current best practice, and that is reasonably likely to achieve the required ESC standard; and

- a good working knowledge of the correct installation, operational and maintenance procedures for the full range of ESC measures used on the site.

Acceptable Solution A9(b)

The construction industry’s method of dealing with workplace safety issues is a good model for the development of an appropriate “chain of command” for the protection of environmental values.  The aim is to produce a fair, reasonable and practicable approach based on environmental risk.

As in workplace safety, the responsibility of environmental protection, and therefore erosion and sediment control, rests with all site personnel, whether or not the work site is the normal place of work of any and all personnel.  Establishing a “chain of command” does not diminish the responsibility of each and every person to take all reasonable and practicable measures to minimise environmental harm resulting from their actions as per their “environmental duty of care”.

Acceptable Solution A10(a)

The exception to this clause is land disturbance necessary to provide access and allow the installation the initial ESC measures.

In general, initial land-disturbing activities should be limited to the establishment of the site compound, site entry/exit points, temporary drainage controls (including drain stabilisation measures), haul road(s), perimeter sediment controls, installation of flow diversion barriers, and any sediment basins/traps required for the first stage of works.

Acceptable Solution A10(b)

“Operational activities” include such things as material stockpiles, storage areas, or concrete waste receptors.

Acceptable Solution A11(a)

It is recognised that it may not be practicable for all stormwater runoff from all areas of site entry/exit paths to be directed to a sediment trap; however, such areas must be limited to the minimum practicable.

SITE MANAGEMENT

Acceptable Solution A12(a)

Where appropriate, an Erosion and Sediment Control Plan is prepared (in accordance with Section G3.3), and where necessary approved by a relevant regulatory authority, prior to commencing any land-disturbing activities.

Acceptable Solution A12(b)

The potential for environmental harm must be assessed by a recognised expert or authority.

Acceptable Solution A12(c)

Refer to A1(a) for a discussion on “potential environmental risk”.

Acceptable Solution A12(d)

Applies to all land-disturbing activities, whether planned or unplanned, and especially to any works that are required to be conducted without an associated Erosion and Sediment Control Plan.

Acceptable Solution A12(d)(iv)

Includes ensuring that the value and use of land/properties adjacent to the development (including roads) are not diminished as a result of work-related soil erosion and sediment runoff.

Acceptable Solution A13

“Responsible ESC personnel” are those persons employed or contracted by the landowner and/or developer as the principal officer(s) responsible for ensuring appropriate application of the planned ESC measures and for the provision of advice in response to unplanned ESC issues.

Acceptable Solution A14(a)

Recommended training requirements are discussed in Section 6.19 of IECA (2008).

Acceptable Solution A14(b)

Necessary disturbance to ESC measures would include the short-term removal of an ESC measure to allow the installation of services under the ESC measure, or to allow vehicular or material access.

Performance Criterion P15

Performance Criteria P15 and P16 require work sites to be appropriately prepared for both current and imminent site conditions.  Compliance with these criteria requires ESCPs to be living documents that remain both effective and flexible, and thus are able to appropriately adapt to changing site conditions.

Acceptable Solution A15(b)

A significant change in site conditions includes:

- unseasonable weather conditions;

- unseasonable stream flow;

- exposure of problematic soil conditions not previously anticipated;

- significant change in construction methodology, staging or programming of earthworks and/or site stabilisation activities;

- significant change in the development design or layout;

- an unprogrammed site shutdown.

Performance Criterion P16

Performance Criteria P15 and P16 require work sites to be appropriately prepared for both current and imminent site conditions.  Compliance with these criteria requires ESCPs to be living documents that remain both effective and flexible, and thus are able to appropriately adapt to changing site conditions.

Acceptable Solution A18(a)

Appropriate identification depends on the level of risk of damage to protected or retained vegetation.  Appropriate identification does not necessarily mean markers, signs or fencing; however, such measures may be appropriate in some areas.

Acceptable Solution A19(b)

AS1940 The storage and handling of flammable and combustible liquids (as amended from time to time).

In addition to the above:

- Impervious bunds must be constructed around all storage areas containing more than 1m3 of petroleum and oil-based products such that the enclosed volume is large enough to contain 110% of the volume held in the largest, individual storage tank.

- On-site personnel involved in the handling and storage of flammable and combustible liquids, including all liquid chemicals, must be appropriately trained and/or supervised, as required in order to allow such personnel to appropriately preform such activities.

Acceptable Solution A19(d)

Current (2008) best practice requires that all reasonable and practicable measures are taken to:

- prevent the release of cement-laden runoff, concrete waste, and chemical products (including petroleum and oil-based products), into an internal or external water body, completed internal drainage systems, or any external drainage system, excluding those on-site drains and water bodies specifically designed to contain and/or treat such material;

- ensure all solid and liquid waste from concrete production, concreting equipment (including delivery and placement vehicles), is fully contained within the property;

- ensure cement residue from work activities is:

- retained on a pervious surface (e.g. a grassed or open soil area, or excavated trench); or

- filtered through a fine-grained, porous, earth embankment; or

- collected and disposed of in a manner that minimise ongoing environmental harm.

Acceptable Solution A19(e)

Current (2008) best practice requires that wherever practicable, the washing of tools and painting equipment is carried out in a manner that:

- complies with current State guidelines, policies and legislation; and

- fully contains any contaminated waste water for later treatment and/or lawful disposal; or

- appropriately filters (e.g. through a fine-grained, porous earth embankment) any contaminated liquid prior to its release from the immediate work area; or

- appropriately infiltrates all contaminated liquid matter into an area of porous grass or open soil.

Acceptable Solution A21(a)

“Sediment and other material” includes clay, silt, sand, gravel, soil, mud, cement and fine-ceramic waste.

Acceptable Solution A21(b)

Sealed surfaces include sealed roads and car parks.

In circumstances where the washing/flushing of sealed surfaces is required, all reasonable and practicable sediment control measures must be employed to prevent, or at least minimise, the release of sediment into receiving waters.  Only those measures that will not cause safety issues or adverse property flooding to third parties shall be employed.

Acceptable Solution A22

“Appropriate consideration” includes taking all reasonable and practicable measures to minimise safety risks.  As a general rule, safety issues take a higher priority than ESC issues; however, this does not mean that the existence of potential safety issues diminishes the ESC standard required of a work site.

Public safety risks include potential damage to public vehicles resulting from the use of inappropriate kerb-inlet sediment traps on public roads. The potential safety risk of a proposed sediment trap to site workers and the public must be given appropriate consideration before its installation, especially those sediment traps located within publicly accessible areas.

Sediment and sediment-laden runoff must not settle or collect on public roadways where such material could result in a traffic or safety hazard.

Performance Criterion P23

The protection of wildlife does not diminish the required ESC standard, or the need to take all reasonable and practicable measures to minimise environmental harm resulting from soil erosion and displaced sediment.

Acceptable Solution A23(c)

Refer to Witheridge (2002) for guidelines on the design of fish-friendly watercourse crossings.

Acceptable Solution A23(b)

Synthetic reinforced fabrics include “plastic” reinforced Erosion Control Blankets, Mats and Meshes.

SITE DISTURBANCE

Acceptable Solution A24(d)

Operational restrictions on the extent and duration of land disturbance, including land clearing only apply when such land disturbance is at risk, or potentially at risk, of erosion by wind, rain or flowing water.

The potential erosion risk is related (in part) to the potential rainfall erosivity as defined in Section 4.4 of IECA (2008). The potential erosion hazard may be identified through the application of an appropriate Erosion Hazard Assessment scheme such as those discussed in Chapter 3 and Appendix F of IECA (2008).

Acceptable Solution A24(e)

The full impact of the removal of deep-rooted vegetation from steep slopes may not be evident for 5 to 10 years, or until such time as the plant root system begins to fail (assuming that the root system remains within the soil profile after removal of the upper portion of the plant).  Planners and designers must appreciate that plants provide many essential roles besides the provision of “scenery”.

Periods of high and extreme erosion potential refers to the variation in the erosion hazard throughout a calender year based on variations in the rainfall erosivity as described in Appendix E of IECA (2008).  Periods of high to extreme erosion potential include:

- periods of high to extreme erosion risk as defined in Section 4.4 of IECA (2008); and

- periods of strong winds sufficient to cause significant dust problems.

Acceptable Solution A25(a)

The extent of unnecessary soil disturbance, including disturbances outside the designated work area, must be minimised at all times.

Wherever reasonable and practicable, land clearing must be limited to the current stage of works.  Current (2008) best practice recommends that land clearing not extend beyond the parameters indicated in Table I11 of IECA (2008).

Table I11 of IECA (2008) does not imply that land clearing should occur to the full extent of these limits, rather than all reasonable and practicable measures are taken to limit land clearing to no more than these limits.  In all cases, land clearing must be limited to the minimum necessary to complete the approved works.

SOIL AND STOCKPILE MANAGEMENT

Performance Criterion A27

Applies to all areas of proposed soil disturbance, including footprint of proposed stockpiles prior to placement of soil within such areas. Does not include any material best described as subsoil.

Acceptable Solution A27(b)

Current (2008) best practice recommendations for the management of topsoil are presented in Table 6.2 of IECA (2008).

Acceptable Solution A28(ii)

The diversion of overbank, stormwater is recommended during those periods when rainfall is possible and the overbank catchment area exceeds 1500m2.

Current (2008) best practice recommendations for the protection of sand and soil stockpiles from the erosive effects of wind and rainfall are presented in Table 4.6.1 of IECA (2008).

Acceptable Solution A28(iv)

Current (2008) best practice recommendations for the selection of an appropriate sediment control system is presented in Table 4.6.2 of IECA (2008).

Short-term stockpiles of erodible material located outside of an appropriate sediment control zone must be covered if it is raining, or if rain is imminent or possible.

Acceptable Solution A29

Dispersive soils normally need to be stabilised (i.e. treated with gypsum or lime depending on desired pH adjustment) and/or buried under a layer of non-dispersive soil prior to placement of channel lining (whether rock, gabion, synthetic material, or concrete), or initiation of revegetation.

Acceptable Solution A30

Within Queensland, guidelines on the management of acid sulfate soils is provided in State Planning Policy 2/02 Guideline: Planning and Managing Development involving Acid Sulfate Soils, and Dear, et al. 2002, Queensland Acid Sulfate Soil Technical Manual – Soil Management Guidelines. Department of Natural Resources and Mines, Indooroopilly, Queensland.

DRAINAGE CONTROL

The intent of this section is to take all reasonable and practicable measures to prevent, or at least minimise, environmental harm and public nuisance resulting from the exposure of soil to the erosive forces of flowing water.  It is not the intent to unfairly burden those performing land-disturbing activities with the cost and inconvenience of installing and maintaining drainage control measures if there is no risk of such environmental harm and public nuisance.

Acceptable Solution A32(a)

Current (2008) best practice construction phase drainage standards are presented in Table 4.3.1 of IECA (2008). Drainage systems must be designed to have a minimum non-erosive hydraulic capacity (excluding 150mm freeboard) in accordance with this table.

Acceptable Solution A32(b)

Construction Drainage Plans are normally prepared for sites with a soil disturbance exceeding 50m2. Further discussion on the requirements of Construction Drainage Plans is presented in Acceptable Solution A11(d).

Acceptable Solution A33(b)

Sandbag flow diversion banks, catch drains, and flow diversion banks are examples of appropriate drainage systems that can be used to divert stormwater around excavations and other soil disturbances.

EROSION CONTROL

The intent of this section is to take all reasonable and practicable measures to prevent, or at least minimise, environmental harm and public nuisance resulting from the exposure of soil, sand, silt, mud or cement to the erosive forces of wind, rain and flowing water. It is not the intent to unfairly burden those performing land-disturbing activities with the cost and inconvenience of installing and maintaining erosion control measures if there is no risk of such environmental harm and public nuisance.

Acceptable Solution A35(a)

Current (2008) best practice (construction phase) land clearing and site rehabilitation standards are presented in Table I11 of IECA (2008). Unless otherwise stated by the relevant regulatory authority, the potential erosion risk is based on the rating outlined in Tables I9 and I10 of IECA (2008).

In addition, all temporary earth banks, flow diversion systems, and off-stream Sediment Basin embankments should be machine-compacted, seeded and mulched within ten (10) days of formation for the purpose of establishing a vegetative cover, unless otherwise stated within an approved Site Stabilisation Plan, Revegetation Plan, or Vegetation Management Plan.

Acceptable Solution A35(b)

Erosion control measures primarily focus on the control of fine sediments such as clay and silt-sized particles. Thus, with respect to the value of “erosion control measures”, potential environmental harm is strongly related to the susceptibility of the receiving waters to environmental harm resulting from turbid runoff (i.e. suspended fine sediments).

Erosion control measures need to be appropriate for the land slope and the expected wind, rain and hydraulic conditions.  Application of effective drainage control measures should help to control hydraulic conditions such that damage to adopted erosion control measures during regular rainfall events is minimised.

Acceptable Solution A35(c)

This clause requires compliance with Performance Criteria P15 and P16.

Acceptable Solution A36(a)

Such a clause shall not reduce the responsibility to apply and maintain, at all times, all necessary sediment control measures.

The minimisation of soil erosion requires the application of effective drainage and erosion control throughout each and all sub-catchments.

Acceptable Solution A37(b)

Compliance with this clause requires:

- soil disturbance within any sub-catchment to be delayed as long as possible, and ideally, not until the principal on-site activities within that area are ready to commence;

- soil disturbance at any given time to be limited to the minimum necessary to perform the required works;

- the extent of unnecessary soil disturbance, including disturbances outside the designated work area, to be minimised.

The stabilisation of non-completed earthworks that are likely to be exposed to rainfall is discussed in Table I11 of IECA (2008).

Compliance with the requirements outlined within Table I11 of IECA (2008) does not diminish the need to apply all reasonable erosion control measures as soon as practicable.

Acceptable Solution A38(c)

Dispersive soils normally need to be stabilised (i.e. treated with gypsum or lime depending on desired pH adjustment) and/or buried under a layer of non-dispersive soil prior to placement of channel lining (whether rock, gabion, synthetic material, or concrete), or initiation of revegetation.

SEDIMENT CONTROL

The intent of this section is to take all reasonable and practicable measures to prevent, or at least minimise, environmental harm and public nuisance resulting from the exposure, placement, or displacement of sediment (including soil, sand, silt, mud and cement). It is not the intent to unfairly burden those performing land-disturbing activities with the cost and inconvenience of installing and maintaining sediment control measures if there is no risk of such environmental harm and public nuisance.

Acceptable Solution A39(a)

Current (2008) best practice (construction phase) sediment control standards are presented in Table 4.5.1 of IECA (2008).

Acceptable Solution A39(b)

Relevant site conditions include the soil type, design flow rate, flow condition (i.e. sheet flow or concentrated flow) and erosion hazard.

Unless otherwise noted within this document, or specified by the regulatory authority, the design storm for off-stream sediment traps (excluding de-watering and instream sediment control measures) must be taken as 0.5 times the 1 in 1 year ARI peak discharge.

The “potential environmental risk” is discussed in Acceptable Solution A1(a), and is summarised in Table 5.1 of IECA (2008).

Acceptable Solution A42(a)

The intent of this clause is to minimise the quantity of water that needs to be de-watered from excavations and trenches.  Thus, if water does not need to be de-watered from such areas, then the clause does not apply.

Acceptable Solution A42(b)

Current (2008) best practice sediment control standards for de-watering activities are outlined in Table 4.5.13 of IECA (2008).

Alternatively, Table 4.5.14 of IECA (2008) presents a water quality standard for de-watering operations based on Nephelometric Turbidity Units (NTU).

Appropriate sediment controls placed down-slope of material stockpiles during the de-watering of such stockpiles are summarised in Table 4.5.14 of IECA (2008).

SITE STABILISATION AND REHABILITATION

Acceptable Solution A43(a)

Site Stabilisation Plans, Landscape Plans, and/or Vegetation Management Plans must show progressive stabilisation of exposed soil for the purposes of erosion control, including but not limited to, all of the following:

- schedule for stabilisation of exposed soil areas; and

- specifications for subsoil and topsoil preparation and application; and

- specification of stabilisation by mulching or other appropriate surface treatment (note, grass seeding without adequate mulching is generally not considered best practice); and

- details on the type and application rate of any tackifiers to be used in the application of mulches (including hydromulch, Bonded Fibre Matrix, and Compost Blankets).

Water Quality Monitoring Programs must document proposed water quality monitoring, and include:

- location of all instream water quality monitoring stations;

- water quality monitoring, sampling, and analysis procedures and standards.

Acceptable Solution A43(b)

Current (2008) best practice site rehabilitation standards are presented in Table I11 of IECA (2008). Unless otherwise stated by the relevant regulatory authority, the potential erosion risk is based on the rating outlined in Tables I9 and I10 of IECA (2008).

Acceptable Solution A44(a)

Temporary revegetation conducted for the purpose of erosion control must be conducted in accordance with a Site Stabilisation Plan, Landscape Plan, Revegetation Plan, or Vegetation Management Plan, where such a plan specifically refers to such activities.

Acceptable Solution A44(b)

The type of permanent vegetation applied to completed earthworks must be compatible with the anticipated long-term land use, current and ongoing erosion risk, environmental requirements (including weed control), and associated components of the site rehabilitation.

Performance Criterion P45

Local environment includes local wildlife.

SITE INSPECTION AND MONITORING

Acceptable Solution A46(b)

Personnel preparing and/or supervising the preparation of the Monitoring and Maintenance Program must, collectively, have the following capabilities:

- an understanding of the local environmental values that could potentially be affected by the proposed works; and

- a good working knowledge of the site’s Erosion and Sediment Control (ESC) issues, and potential environmental impacts, that is commensurate with the complexity of the site and the degree of environmental risk; and

- a good working knowledge of current best practice Erosion and Sediment Control measures appropriate for the given site conditions and type of works; and

- a good working knowledge of the correct installation, operational and maintenance procedures for the full range of ESC measures used on the site.

Refer to A1(a) for discussion on “potential environmental risk “.

Acceptable Solution A47(a)

Discussion on scheduling and conducting site inspections by internal and external parties is provided in Chapter 7 of IECA (2008).

In those instances where specific site monitoring stations are identified within the Monitoring and Maintenance Program, then:

- during periods of water discharge from the site, water quality samples are collected at each monitoring station at least once on each calendar day until such discharge stops; and

- a minimum of 3 water samples are taken and analysed, and the average result used to determine quality.

Current (2008) best-practice procedures for “high-risk” sites, requires regular ESC audits to be:

- undertaken by a person suitably qualified and experienced in erosion and sediment control that can be verified by an independent third-party (this person must not be an employee or agent of the principal contractor); and

- conducted on the next business day following a rainfall event in which greater than 10 mm of rainfall has been recorded by the Bureau of Meteorology rain gauge nearest to the site; and

- conducted at intervals of not more than one (1) calendar month commencing from the day of site disturbance until all disturbed areas have been adequately stabilised against erosion to the acceptance of the relevant regulatory authority; and

- conducted using an appropriate Site Inspection Checklist.

“High-risk sites” are work sites that:

- satisfy the requirements of a high-risk site as defined by either the State or local government; or

- satisfy the requirements of those risk categories greater than high-risk (such as extreme-risk) where such categories have been defined (i.e. score a hazard rating equal to or greater than the “critical hazard value”).

Discussion on the assessment of erosion hazard and site risk assessment is presented in Chapter 3 and Appendix F of IECA (2008).

ESC audits must include, as a minimum:

- copies of all original Site Inspection Checklists; and

- non-conformance and corrective action reports;

- sediment basin water quality and site discharge water quality monitoring results;

- a plan showing the areas of completed soil stabilisation; and

- rainfall records including date and rainfall depth.

Acceptable Solution A48

Discussion on scheduling and conducting of site inspections is provided in Chapter 7 of IECA (2008).

SITE MAINTENANCE

Performance Criterion P49

Proper working order includes maintaining the required hydraulic capacity and operational effectiveness.

Acceptable Solution A49(b)

Current (2008) best practice requirements for the maintenance of sediment control devices requires these devices to be maintained and made fully operational as soon as reasonable and practicable in accordance with Table 6.1 of IECA (2008).

Reference

IECA 2008, Best Practice Erosion and Sediment Control. International Erosion Control Association (Australasia), Picton NSW.